Gyro code working without yaw

#include <Wire.h>

const int HMC_ADDR = 0x1E; // HMC5883L I2C address

void setup() {

Wire.begin();

Serial.begin(115200);

// Initialize HMC5883L

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x00); // Configuration Register A

Wire.write(0x70); // 8-average, 15 Hz default, normal measurement

Wire.endTransmission();

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x01); // Configuration Register B

Wire.write(0xA0); // Gain = 5

Wire.endTransmission();

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x02); // Mode Register

Wire.write(0x00); // Continuous measurement mode

Wire.endTransmission();

Serial.println("HMC5883L ready");

}

void loop() {

int16_t X, Y, Z;

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x03); // Data output X MSB register

Wire.endTransmission();

Wire.requestFrom(HMC_ADDR, 6);

if (Wire.available() == 6) {

X = Wire.read() << 8 | Wire.read();

Z = Wire.read() << 8 | Wire.read(); // Z-axis (optional)

Y = Wire.read() << 8 | Wire.read();

}

// Calculate heading (yaw) in degrees

float heading = atan2((float)Y, (float)X) * 180.0 / 3.14159;

// Normalize to 0–360

if (heading < 0) heading += 360;

Serial.println(heading);

delay(50); // 20 Hz update

full and final code showing heading and yawrate  which is working

#include <Wire.h>

const int HMC_ADDR = 0x1E; // HMC5883L I2C address

float lastHeading = 0;

float lastYawRate = 0;

unsigned long lastTime = 0;

const float alpha = 0.2;   // smoothing factor 0–1

const float deadband = 1.0; // ignore small heading changes in degrees

void setup() {

Wire.begin();

Serial.begin(115200);

// Initialize HMC5883L

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x00); // Configuration Register A

Wire.write(0x70); // 8-average, 15Hz, normal measurement

Wire.endTransmission();

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x01); // Configuration Register B

Wire.write(0xA0); // Gain = 5

Wire.endTransmission();

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x02); // Mode Register

Wire.write(0x00); // Continuous measurement

Wire.endTransmission();

delay(100);

lastTime = millis();

}

void loop() {

int16_t X, Y, Z;

// Read magnetometer

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x03); // Data output X MSB

Wire.endTransmission();

Wire.requestFrom(HMC_ADDR, 6);

if(Wire.available() == 6){

X = Wire.read() << 8 | Wire.read();

Z = Wire.read() << 8 | Wire.read(); // optional

Y = Wire.read() << 8 | Wire.read();

} else {

return; // skip if data unavailable

}

// Compute heading

float heading = atan2((float)Y, (float)X) * 180.0 / 3.14159;

if(heading < 0) heading += 360;

// Compute time delta

unsigned long now = millis();

float dt = (now - lastTime) / 1000.0; // seconds

lastTime = now;

// Compute delta heading with wrap-around

float delta = heading - lastHeading;

if(delta > 180) delta -= 360;

if(delta < -180) delta += 360;

// Deadband: ignore tiny fluctuations

if(abs(delta) < deadband) delta = 0;

// Compute raw yaw rate

float yawRateRaw = delta / dt;

// Apply simple low-pass filter for smoothing

float yawRate = alpha * yawRateRaw + (1 - alpha) * lastYawRate;

lastYawRate = yawRate;

// Update last heading

lastHeading = heading;

// Print results

Serial.print("Heading: ");

Serial.print(heading, 2);

Serial.print("  YawRate: ");

Serial.println(yawRate, 2);

delay(50); // ~20 Hz update

code

#include <Wire.h>

const int HMC_ADDR = 0x1E; // HMC5883L I2C address

float lastHeading = 0;

float lastYawRate = 0;

unsigned long lastTime = 0;

const float alpha = 0.2;   // smoothing factor 0–1

const float deadband = 1.0; // ignore small heading changes in degrees

void setup() {

Wire.begin();

Serial.begin(115200);

// Initialize HMC5883L

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x00); // Configuration Register A

Wire.write(0x70); // 8-average, 15Hz, normal measurement

Wire.endTransmission();

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x01); // Configuration Register B

Wire.write(0xA0); // Gain = 5

Wire.endTransmission();

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x02); // Mode Register

Wire.write(0x00); // Continuous measurement

Wire.endTransmission();

delay(100);

lastTime = millis();

}

void loop() {

int16_t X, Y, Z;

// Read magnetometer

Wire.beginTransmission(HMC_ADDR);

Wire.write(0x03); // Data output X MSB

Wire.endTransmission();

Wire.requestFrom(HMC_ADDR, 6);

if(Wire.available() == 6){

X = Wire.read() << 8 | Wire.read();

Z = Wire.read() << 8 | Wire.read(); // optional

Y = Wire.read() << 8 | Wire.read();

} else {

return; // skip if data unavailable

}

// Compute heading

float heading = atan2((float)Y, (float)X) * 180.0 / 3.14159;

if(heading < 0) heading += 360;

// Compute time delta

unsigned long now = millis();

float dt = (now - lastTime) / 1000.0; // seconds

lastTime = now;

// Compute delta heading with wrap-around

float delta = heading - lastHeading;

if(delta > 180) delta -= 360;

if(delta < -180) delta += 360;

// Deadband: ignore tiny fluctuations

if(abs(delta) < deadband) delta = 0;

// Compute raw yaw rate

float yawRateRaw = delta / dt;

// Apply simple low-pass filter for smoothing

float yawRate = alpha * yawRateRaw + (1 - alpha) * lastYawRate;

lastYawRate = yawRate;

// Update last heading

lastHeading = heading;

// Print results

Serial.print("Heading: ");

Serial.print(heading, 2);

Serial.print("  YawRate: ");

Serial.println(yawRate, 2);

delay(50); // ~20 Hz update

}