# Lab Report: AlphaBot Yaw Measurement Using GY-85 Magnetometer

Course / Module: Autonomous Mobile Systems Lab – Hochschule Hamm-Lippstadt
Date: 10.03.2026
Authors: [Your Name]

## 1. Introduction

The goal of this experiment was to measure the yaw rate of an AlphaBot robot using the GY-85 IMU module, and compare it with the ground truth provided by a TopCon total station. The project is part of a larger effort to develop autonomous navigation and mapping using low-cost sensors.

Due to voltage constraints (3.3 V available on the setup), the gyro inside the GY-85 could not be used, so the magnetometer (HMC5883L) was employed to estimate robot heading, from which the yaw rate could be approximated.

## 2. Materials and Equipment

- AlphaBot robotic platform

- GY-85 IMU sensor module (HMC5883L, MPU6050, BMP180)

- Arduino Uno

- Arduino IDE (latest version)

- MATLAB (for later processing)

- USB connection for serial communication

## 3. Method

### 3.1 Sensor Selection and Setup

- The GY-85 was powered with 3.3 V, which is sufficient for the magnetometer (HMC5883L) but insufficient for the MPU6050 gyro.

- The magnetometer provides absolute heading (0–360°) in the horizontal plane.

- Arduino communicates with the sensor via I2C:

| GY-85 Pin | Arduino Pin |
| --- | --- |
| VCC | 3.3 V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

### 3.2 Arduino Code Implementation

- Initialize the HMC5883L magnetometer: set measurement mode, averaging, and gain registers.

- Read X, Y, Z magnetic field components from the sensor every 50 ms.

- Compute the heading angle:

heading=atan2(Y,X)×180π\text{heading} = \text{atan2}(Y, X) \times \frac{180}{\pi}heading=atan2(Y,X)×π180​

- Normalized to 0–360°

- Compute approximate yaw rate:

delta=headingcurrent−headingprevious\text{delta} = \text{heading}_{current} - \text{heading}_{previous}delta=headingcurrent​−headingprevious​

- Apply wrap-around correction if delta > 180° or delta < -180°

- Apply deadband to ignore small fluctuations (<1°)

- Apply low-pass filter to smooth noise:

yawRate=α⋅yawRateRaw+(1−α)⋅yawRatePrevious\text{yawRate} = \alpha \cdot \text{yawRateRaw} + (1-\alpha) \cdot \text{yawRatePrevious}yawRate=α⋅yawRateRaw+(1−α)⋅yawRatePrevious

- Print heading and yaw rate over serial for real-time observation.

### 3.3 Observations

- When the robot is stationary, the measured yaw rate is near 0°/s, indicating effective noise suppression.

- When the robot rotates, the yaw rate updates smoothly and corresponds to the direction of rotation.

- Raw magnetometer readings fluctuate naturally due to sensor noise, which is mitigated by deadband and low-pass filtering.

## 4. Results

| Condition | Heading (deg) | Yaw Rate (deg/s) |
| --- | --- | --- |
| Robot stationary | ~180 | ~0 |
| Small rotation | 185 → 190 | ~20–40 |
| Faster rotation | 190 → 230 | ~80–100 |

- The Arduino implementation successfully provides realistic yaw rate values derived from magnetometer heading.

- Noise spikes while stationary are minimized via deadband and smoothing.

## 5. Discussion

- The yaw rate is approximate, as it is derived from heading differences, not a gyro.

- True instantaneous yaw rates require a working MPU6050 gyro, which is not possible at 3.3 V.

- This approach is sufficient for slow or moderate rotations, mapping, and comparison with TopCon data.

- The method can later be integrated into MATLAB for real-time plotting and comparison.

## 6. Conclusion

- The AlphaBot was successfully interfaced with the GY-85 magnetometer via Arduino.

- Heading and yaw rate were calculated reliably, with noise filtering applied.

- This forms a working base for experimentally comparing measured yaw rate with TopCon ground truth.

- Limitations include high-frequency yaw rate fluctuations and dependency on magnetic environment.

## 7. Future Work

- Integrate MATLAB code for real-time plotting and data logging.

- Compare with TopCon positional and yaw measurements.

- If possible, use a 5 V-powered gyro for more accurate yaw rate measurement.

### References

- HMC5883L Datasheet: Honeywell, 3-Axis Magnetometer

- AlphaBot Platform Documentation – Hochschule Hamm-Lippstadt

- Arduino Wire Library Documentation