Arduino Code for 5 IR Sensors (Line Follower)

#define PWMA 6

#define AIN1 A1

#define AIN2 A0

#define PWMB 5

#define BIN1 A2

#define BIN2 A3

#define S1 2

#define S2 3

#define S3 4

#define S4 7

#define S5 8

int speed = 120;

void setup()

{

pinMode(PWMA, OUTPUT);

pinMode(PWMB, OUTPUT);

pinMode(AIN1, OUTPUT);

pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT);

pinMode(BIN2, OUTPUT);

pinMode(S1, INPUT);

pinMode(S2, INPUT);

pinMode(S3, INPUT);

pinMode(S4, INPUT);

pinMode(S5, INPUT);

}

void loop()

{

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

if(s3 == LOW)

{

forward();

}

else if(s2 == LOW)

{

left();

}

else if(s1 == LOW)

{

sharpLeft();

}

else if(s4 == LOW)

{

right();

}

else if(s5 == LOW)

{

sharpRight();

}

else

{

stop();

}

}

void forward()

{

analogWrite(PWMA, speed);

analogWrite(PWMB, speed);

digitalWrite(AIN1, LOW);

digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, LOW);

digitalWrite(BIN2, HIGH);

}

void left()

{

analogWrite(PWMA, 80);

analogWrite(PWMB, 120);

}

void right()

{

analogWrite(PWMA, 120);

analogWrite(PWMB, 80);

}

void sharpLeft()

{

analogWrite(PWMA, 0);

analogWrite(PWMB, 120);

}

void sharpRight()

{

analogWrite(PWMA, 120);

analogWrite(PWMB, 0);

}

void stop()

{

analogWrite(PWMA, 0);

analogWrite(PWMB, 0);

}

Arduino PID Line Follower Code (5 IR Sensors)

#define PWMA 6

#define PWMB 5

#define AIN1 A1

#define AIN2 A0

#define BIN1 A2

#define BIN2 A3

#define S1 2

#define S2 3

#define S3 4

#define S4 7

#define S5 8

int baseSpeed = 120;

float Kp = 25;

float Ki = 0;

float Kd = 15;

int error = 0;

int lastError = 0;

int integral = 0;

void setup()

{

pinMode(PWMA, OUTPUT);

pinMode(PWMB, OUTPUT);

pinMode(AIN1, OUTPUT);

pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT);

pinMode(BIN2, OUTPUT);

pinMode(S1, INPUT);

pinMode(S2, INPUT);

pinMode(S3, INPUT);

pinMode(S4, INPUT);

pinMode(S5, INPUT);

forward();

}

void loop()

{

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

if(s1 == LOW) error = -2;

else if(s2 == LOW) error = -1;

else if(s3 == LOW) error = 0;

else if(s4 == LOW) error = 1;

else if(s5 == LOW) error = 2;

integral = integral + error;

int derivative = error - lastError;

int correction = Kp * error + Ki * integral + Kd * derivative;

int leftMotorSpeed = baseSpeed + correction;

int rightMotorSpeed = baseSpeed - correction;

leftMotorSpeed = constrain(leftMotorSpeed, 0, 255);

rightMotorSpeed = constrain(rightMotorSpeed, 0, 255);

analogWrite(PWMA, leftMotorSpeed);

analogWrite(PWMB, rightMotorSpeed);

lastError = error;

}

void forward()

{

digitalWrite(AIN1, LOW);

digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, LOW);

digitalWrite(BIN2, HIGH);

}

Advanced IR5 PID Line Follower Code

#define PWMA 6

#define PWMB 5

#define AIN1 A1

#define AIN2 A0

#define BIN1 A2

#define BIN2 A3

#define S1 2

#define S2 3

#define S3 4

#define S4 7

#define S5 8

int baseSpeed = 130;

float Kp = 25;

float Ki = 0;

float Kd = 15;

int error = 0;

int lastError = 0;

int integral = 0;

void setup()

{

pinMode(PWMA, OUTPUT);

pinMode(PWMB, OUTPUT);

pinMode(AIN1, OUTPUT);

pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT);

pinMode(BIN2, OUTPUT);

pinMode(S1, INPUT);

pinMode(S2, INPUT);

pinMode(S3, INPUT);

pinMode(S4, INPUT);

pinMode(S5, INPUT);

forwardDirection();

}

void loop()

{

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

// LINE LOST

if(s1 && s2 && s3 && s4 && s5)

{

if(lastError < 0)

sharpLeft();

else

sharpRight();

return;

}

// ERROR CALCULATION

if(s1 == LOW) error = -2;

else if(s2 == LOW) error = -1;

else if(s3 == LOW) error = 0;

else if(s4 == LOW) error = 1;

else if(s5 == LOW) error = 2;

integral += error;

int derivative = error - lastError;

int correction = Kp*error + Ki*integral + Kd*derivative;

int leftSpeed = baseSpeed + correction;

int rightSpeed = baseSpeed - correction;

leftSpeed = constrain(leftSpeed,0,255);

rightSpeed = constrain(rightSpeed,0,255);

analogWrite(PWMA,leftSpeed);

analogWrite(PWMB,rightSpeed);

lastError = error;

}

void forwardDirection()

{

digitalWrite(AIN1, LOW);

digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, LOW);

digitalWrite(BIN2, HIGH);

}

void sharpLeft()

{

analogWrite(PWMA, 0);

analogWrite(PWMB, 140);

}

void sharpRight()

{

analogWrite(PWMA, 140);

analogWrite(PWMB, 0);

}

Arduino PID Line Follower – Stop on White

#define PWMA 6

#define PWMB 5

#define AIN1 A1

#define AIN2 A0

#define BIN1 A2

#define BIN2 A3

#define S1 2

#define S2 3

#define S3 4

#define S4 7

#define S5 8

int baseSpeed = 130;

float Kp = 25;

float Ki = 0;

float Kd = 15;

int error = 0;

int lastError = 0;

int integral = 0;

void setup()

{

pinMode(PWMA, OUTPUT);

pinMode(PWMB, OUTPUT);

pinMode(AIN1, OUTPUT);

pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT);

pinMode(BIN2, OUTPUT);

pinMode(S1, INPUT);

pinMode(S2, INPUT);

pinMode(S3, INPUT);

pinMode(S4, INPUT);

pinMode(S5, INPUT);

forwardDirection();

}

void loop()

{

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

// LINE LOST – STOP ROBOT

if(s1 && s2 && s3 && s4 && s5)

{

stopRobot();

return;

}

// ERROR CALCULATION

if(s1 == LOW) error = -2;

else if(s2 == LOW) error = -1;

else if(s3 == LOW) error = 0;

else if(s4 == LOW) error = 1;

else if(s5 == LOW) error = 2;

integral += error;

int derivative = error - lastError;

int correction = Kp*error + Ki*integral + Kd*derivative;

int leftSpeed = baseSpeed + correction;

int rightSpeed = baseSpeed - correction;

leftSpeed = constrain(leftSpeed,0,255);

rightSpeed = constrain(rightSpeed,0,255);

analogWrite(PWMA,leftSpeed);

analogWrite(PWMB,rightSpeed);

lastError = error;

}

void forwardDirection()

{

digitalWrite(AIN1, LOW);

digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, LOW);

digitalWrite(BIN2, HIGH);

}

void stopRobot()

{

analogWrite(PWMA,0);

analogWrite(PWMB,0);

digitalWrite(AIN1,LOW);

digitalWrite(AIN2,LOW);

digitalWrite(BIN1,LOW);

digitalWrite(BIN2,LOW);

}

Working

#include "TRSensors.h"

#define NUM_SENSORS 5

TRSensors trs;

unsigned int sensorValues[NUM_SENSORS];

unsigned int last_proportional = 0;

long integral = 0;

// Motor pins

#define PWMA 6

#define AIN1 A1

#define AIN2 A0

#define PWMB 5

#define BIN1 A2

#define BIN2 A3

// PID constants

const float Kp = 0.05;

const float Ki = 0.0005;

const float Kd = 0.2;

const int MAX_SPEED = 100;

// Threshold for white line detection (experimentally)

const int WHITE_THRESHOLD = 800; // adjust based on your sensor readings

void setup() {

Serial.begin(115200);

pinMode(PWMA, OUTPUT);

pinMode(AIN1, OUTPUT);

pinMode(AIN2, OUTPUT);

pinMode(PWMB, OUTPUT);

pinMode(BIN1, OUTPUT);

pinMode(BIN2, OUTPUT);

// Stop motors initially

analogWrite(PWMA, 0);

analogWrite(PWMB, 0);

digitalWrite(AIN1, LOW);

digitalWrite(AIN2, LOW);

digitalWrite(BIN1, LOW);

digitalWrite(BIN2, LOW);

// Calibrate sensors (slowly move over black and white surfaces)

for (int i = 0; i < 400; i++) {

trs.calibrate();

delay(10);

}

Serial.println("Calibration done!");

}

void loop() {

trs.AnalogRead(sensorValues);

// Check for white line detection

int maxSensor = 0;

for (int i = 0; i < NUM_SENSORS; i++) {

if (sensorValues[i] > maxSensor) maxSensor = sensorValues[i];

}

if (maxSensor >= WHITE_THRESHOLD) {

// White line detected → stop motors

analogWrite(PWMA, 0);

analogWrite(PWMB, 0);

digitalWrite(AIN1, LOW);

digitalWrite(AIN2, LOW);

digitalWrite(BIN1, LOW);

digitalWrite(BIN2, LOW);

Serial.println("White line detected! Stopping.");

while (1); // stop forever

}

// PID line following on black background

unsigned int position = trs.readLine(sensorValues);

int proportional = (int)position - 2000;

int derivative = proportional - last_proportional;

integral += proportional;

if (integral > 50000) integral = 50000;

if (integral < -50000) integral = -50000;

last_proportional = proportional;

float correction = Kp * proportional + Ki * integral + Kd * derivative;

int leftSpeed = constrain(MAX_SPEED + correction, 0, MAX_SPEED);

int rightSpeed = constrain(MAX_SPEED - correction, 0, MAX_SPEED);

// LEFT MOTOR forward

analogWrite(PWMA, leftSpeed);

digitalWrite(AIN1, HIGH); // adjust if motor runs backward

digitalWrite(AIN2, LOW);

// RIGHT MOTOR forward

analogWrite(PWMB, rightSpeed);

digitalWrite(BIN1, HIGH); // adjust if motor runs backward

digitalWrite(BIN2, LOW);

}