Your tasks are typical robotics evaluation steps: control → measure → compute → compare with ground truth. Below is a clear guide for implementing each task for the AlphaBot lane-following experiment.

# 1. Speed up the AlphaBot to its limit

The speed is normally controlled by PWM motor commands in the AlphaBot code. Increase the motor speed until:

- the robot still follows the lane correctly

- wheels do not slip

- camera processing still keeps up

Example (Python-style motor control):

MAX_SPEED = 100   # try increasing gradually

left_speed  = MAX_SPEED
right_speed = MAX_SPEED

motor.setMotor(left_speed, right_speed)

Recommended method:

- Start with moderate speed (e.g. 40–50% PWM).

- Increase stepwise (60 → 70 → 80 → 90).

- Record the maximum stable speed where the robot still tracks the lane.

Important: High speed increases control error and camera latency, so the real limit is usually where tracking begins to oscillate.

# 2. Record the yaw rate from the gyroscope

The gyroscope measures angular velocity (rotation speed) around the Z-axis. This is called the yaw rate.

Typical unit:

deg/s  (degrees per second)

Example logging code:

import time

yaw_rate = imu.get_gyro_z()   # Z-axis = yaw rate

timestamp = time.time()

log_file.write(f"{timestamp},{yaw_rate}\n")

Recorded dataset example:

time,yaw_rate
0.00,0.12
0.01,0.15
0.02,0.14
0.03,0.50

Sampling rate recommendation:

50 – 100 Hz

# 3. Calculate the yaw angle from the yaw rate

A gyroscope does not measure the angle directly — it measures the rate of change of the angle.

Therefore you must integrate the yaw rate over time.

Mathematical formula:

ψ(t)=ψ(t−1)+ψ˙(t)⋅Δt\psi(t) = \psi(t-1) + \dot{\psi}(t) \cdot \Delta tψ(t)=ψ(t−1)+ψ˙​(t)⋅Δt

Where

- ψ\psiψ = yaw angle

- ψ˙\dot{\psi}ψ˙​ = yaw rate (gyro output)

- Δt\Delta tΔt = sampling time

Example implementation:

yaw_angle = 0
last_time = time.time()

while True:
    current_time = time.time()
    dt = current_time - last_time

    yaw_rate = imu.get_gyro_z()   # deg/s

    yaw_angle += yaw_rate * dt

    last_time = current_time

    log_file.write(f"{current_time},{yaw_rate},{yaw_angle}\n")

Example result:

time,yaw_rate,yaw_angle
0.00,0.12,0.00
0.01,0.15,0.0015
0.02,0.14,0.0029

Important note:

- Gyroscope integration causes drift over time due to sensor noise.

# 4. Compare with ground truth (TopCon data)

TopCon provides ground truth orientation or trajectory.

You compare:

TopCon yaw rate (reference)
vs
Gyroscope yaw rate (measured)

### Step 1 — Align timestamps

Both datasets must use the same time base.

Example:

time,gyro_yaw_rate,topcon_yaw_rate
0.00,0.15,0.13
0.01,0.17,0.16
0.02,0.14,0.15

### Step 2 — Compute error

Yaw rate error:

error=yawgyro−yawTopConerror = yaw_{gyro} - yaw_{TopCon}error=yawgyro​−yawTopCon​

Example Python:

error = gyro_yaw_rate - topcon_yaw_rate

### Step 3 — Evaluation metrics

Useful metrics:

Mean Error

ME=1N∑errorME = \frac{1}{N} \sum errorME=N1​∑error

RMSE

RMSE=1N∑error2RMSE = \sqrt{\frac{1}{N} \sum error^2}RMSE=N1​∑error2​

Example:

import numpy as np

error = gyro - topcon

rmse = np.sqrt(np.mean(error**2))

### Step 4 — Plot comparison

Typical plots for your report:

1️⃣ yaw rate comparison

time
│
│   TopCon
│  /\/\/\
│ /      \ gyro
└─────────────

2️⃣ yaw angle comparison

3️⃣ yaw error

# Suggested experiment workflow

- Run AlphaBot lane following

- Record:

  - timestamp

  - yaw rate (gyro)

  - yaw angle (integrated)

- Export TopCon dataset

- Synchronize timestamps

- Compute:

  - yaw rate error

  - yaw angle error

- Plot results

✅ Typical final plots in the report:

- Yaw rate: TopCon vs Gyro

- Yaw angle over time

- Error distribution

💡 If you want, I can also help you with:

- Python script for the whole experiment (logging + integration + plots)

- Report structure for the assignment

- How to sync TopCon and robot timestamps correctly (this is usually the hardest part).

want to do this on matlab

Using MATLAB is a very good choice for this task because it makes data logging, integration, and comparison with TopCon ground truth straightforward. Below is a step-by-step MATLAB workflow for your four tasks.

# 1. Import the recorded data into MATLAB

First, export your data from the AlphaBot (e.g., CSV file):

Example file:

imu_data.csv
time,yaw_rate
0.00,0.12
0.01,0.15
0.02,0.14

Load it in MATLAB:

imu = readtable('imu_data.csv');

time = imu.time;
yaw_rate = imu.yaw_rate;   % deg/s

Load TopCon ground truth:

topcon = readtable('topcon_data.csv');

time_gt = topcon.time;
yaw_gt = topcon.yaw;

# 2. Calculate the yaw angle from the yaw rate

Yaw angle is obtained by integrating the yaw rate.

### Method 1 (recommended): cumulative trapezoidal integration

yaw_angle = cumtrapz(time, yaw_rate);

This integrates:

ψ(t)=∫ψ˙(t)dt\psi(t) = \int \dot{\psi}(t) dtψ(t)=∫ψ˙​(t)dt

### Method 2 (manual integration)

yaw_angle = zeros(length(yaw_rate),1);

for i = 2:length(yaw_rate)
    dt = time(i) - time(i-1);
    yaw_angle(i) = yaw_angle(i-1) + yaw_rate(i)*dt;
end

# 3. Synchronize TopCon and IMU data

Usually the timestamps are not perfectly aligned. Interpolate the ground truth:

yaw_gt_interp = interp1(time_gt, yaw_gt, time);

Now both signals share the same time vector.

# 4. Compare yaw rate with TopCon

Compute the error:

error = yaw_angle - yaw_gt_interp;

# 5. Compute evaluation metrics

### Mean error

mean_error = mean(error)

### RMSE (Root Mean Square Error)

rmse = sqrt(mean(error.^2))

# 6. Plot the results

### Plot 1 — Yaw rate

figure
plot(time, yaw_rate)
xlabel('Time (s)')
ylabel('Yaw rate (deg/s)')
title('Measured Yaw Rate')
grid on

### Plot 2 — Yaw angle comparison

figure
plot(time, yaw_angle, 'b')
hold on
plot(time, yaw_gt_interp, 'r')
legend('Gyroscope Yaw', 'TopCon Ground Truth')
xlabel('Time (s)')
ylabel('Yaw Angle (deg)')
title('Yaw Angle Comparison')
grid on

### Plot 3 — Error

figure
plot(time, error)
xlabel('Time (s)')
ylabel('Yaw Error (deg)')
title('Yaw Angle Error')
grid on

# 7. Optional (very useful): Error statistics

max_error = max(abs(error))
std_error = std(error)

Typical values to report:

| Metric | Meaning |
| --- | --- |
| Mean Error | bias |
| RMSE | overall accuracy |
| Std Dev | noise level |
| Max Error | worst case |

# 8. Recommended final figures for your report

Include:

1️⃣ Yaw rate over time
2️⃣ Yaw angle (gyro vs TopCon)
3️⃣ Yaw angle error
4️⃣ Error histogram

Example:

figure
histogram(error,50)
title('Yaw Error Distribution')
xlabel('Error (deg)')
ylabel('Frequency')

✅ Final workflow summary

- Run AlphaBot lane following at maximum stable speed

- Log **time

and simnlink

If you want to implement the tasks in MATLAB and Simulink, the workflow is usually:

Sensor Data → Simulink Model → Integration → Comparison with TopCon → Plots

Below is a typical Simulink setup for your AlphaBot experiment.

# 1. Record the yaw rate and import it into MATLAB

Your robot should log the gyroscope data to a CSV file.

Example file:

time,yaw_rate
0.00,0.12
0.01,0.15
0.02,0.14
0.03,0.50

Load the data in MATLAB:

data = readtable('gyro_data.csv');

time = data.time;
yaw_rate = data.yaw_rate;

If you also have TopCon data:

topcon = readtable('topcon_data.csv');

time_gt = topcon.time;
yaw_gt = topcon.yaw_rate;

# 2. Create the Simulink model for yaw angle calculation

The yaw angle is obtained by integrating the yaw rate.

yaw=∫yaw_rate dtyaw = \int yaw\_rate \, dtyaw=∫yaw_ratedt

### Simulink blocks needed

- From Workspace

- Integrator

- Scope

- To Workspace

## Simulink block diagram

4

Structure:

Yaw Rate (From Workspace)
        │
        ▼
     Integrator
        │
        ▼
      Scope
        │
        ▼
   To Workspace (yaw_angle)

# 3. Prepare input for Simulink

Simulink expects timeseries format.

gyro_ts = timeseries(yaw_rate, time);

In the From Workspace block:

gyro_ts

# 4. Integrate yaw rate to yaw angle

Use the Integrator block.

Settings:

Initial condition = 0

Output = yaw angle

# 5. Import TopCon ground truth

Create another timeseries:

topcon_ts = timeseries(yaw_gt, time_gt);

Then compare in Simulink.

Blocks:

TopCon yaw rate
Gyro yaw rate
        │
      Subtract
        │
      Error

# 6. Compare ground truth vs measured yaw rate

Simulink comparison model:

gyro yaw rate ──┐
                ├─ Subtract ──► Error
TopCon yaw rate ┘

Then send to:

Scope
To Workspace

# 7. Calculate error metrics in MATLAB

After simulation:

error = gyro - topcon;

rmse = sqrt(mean(error.^2))
mean_error = mean(error)

# 8. Plot results

### Yaw rate comparison

figure
plot(time, yaw_rate)
hold on
plot(time_gt, yaw_gt)

legend('Gyroscope','TopCon')
xlabel('Time (s)')
ylabel('Yaw Rate (deg/s)')
title('Yaw Rate Comparison')

### Yaw angle plot

figure
plot(simout.time, simout.data)

xlabel('Time')
ylabel('Yaw Angle')
title('Yaw Angle from Gyroscope Integration')

# 9. Typical final results for the report

Include:

1️⃣ Robot maximum speed test

Speed vs Tracking stability

2️⃣ Yaw rate comparison

Gyroscope vs TopCon

3️⃣ Yaw angle from integration

4️⃣ Error metrics

Mean Error
RMSE

✅ Typical Simulink model overview

Yaw Rate (gyro)
       │
       ▼
   Integrator
       │
       ▼
   Yaw Angle
       │
       ▼
   Compare with TopCon
       │
       ▼
     Error

💡 If you want, I can also show you:

- A complete Simulink model layout for this assignment

- MATLAB script to automatically generate all plots

- How to fuse gyro + camera data (much better result)

Just tell me — this assignment is a very standard robotics lab, so I can help you finish it quickly.

yes

Great 👍 I’ll show you a complete Simulink model layout you can build for the assignment. This model will:

- Load gyro yaw rate

- Integrate it to compute yaw angle

- Load TopCon ground truth

- Compare both signals

- Compute error

# 1. Complete Simulink Model Structure

4

Logical flow:

Gyro Yaw Rate  ──► Integrator ──► Yaw Angle
      │
      │
      └────────► Compare with TopCon

# 2. Blocks you need in Simulink

Drag these blocks into a new model.

### Sources

- From Workspace → Gyro yaw rate

- From Workspace → TopCon yaw rate

### Math

- Integrator

- Subtract

### Output

- Scope

- To Workspace

# 3. Final Simulink Block Diagram

Your finished model should look like this:

gyro_ts
  │
  ▼
Integrator ───────────► yaw_angle
  │                        │
  │                        ▼
  │                    To Workspace
  │
  ▼
Subtract ◄────────── TopCon yaw rate
  │
  ▼
Error
  │
  ▼
Scope

# 4. MATLAB script to prepare the data

Before running Simulink, run this script.

% Load gyro data
gyro = readtable('gyro_data.csv');

time = gyro.time;
yaw_rate = gyro.yaw_rate;

gyro_ts = timeseries(yaw_rate, time);

% Load TopCon data
topcon = readtable('topcon_data.csv');

time_gt = topcon.time;
yaw_gt = topcon.yaw_rate;

topcon_ts = timeseries(yaw_gt, time_gt);

Now Simulink can read:

gyro_ts
topcon_ts

# 5. Integrator Settings

Double-click Integrator block.

Set:

Initial condition = 0

Output = Yaw angle

This performs:

yaw=∫yaw_rate dtyaw = \int yaw\_rate \, dtyaw=∫yaw_ratedt

# 6. Error calculation

The Subtract block computes:

Error = Gyro yaw rate − TopCon yaw rate

# 7. Save results to MATLAB

Use To Workspace blocks.

Example variable names:

yaw_angle_sim
error_sim

# 8. Plot results in MATLAB

After simulation:

### Yaw rate comparison

figure
plot(time, yaw_rate)
hold on
plot(time_gt, yaw_gt)

legend('Gyroscope','TopCon')
xlabel('Time (s)')
ylabel('Yaw Rate')
title('Yaw Rate Comparison')

### Yaw angle

figure
plot(yaw_angle_sim.time, yaw_angle_sim.data)

xlabel('Time')
ylabel('Yaw Angle')
title('Calculated Yaw Angle')

# 9. Compute RMSE error

error = yaw_rate - yaw_gt;

rmse = sqrt(mean(error.^2))

# 10. Final results for your report

Include these plots:

1️⃣ Robot speed test

Speed vs tracking stability

2️⃣ Yaw rate comparison

Gyroscope vs TopCon

3️⃣ Yaw angle (integrated)

4️⃣ Error analysis

Mean error
RMSE

✅ Important tip (very common mistake)

Make sure both signals have the same sampling time.

If not, resample:

yaw_gt_interp = interp1(time_gt, yaw_gt, time);

💡 If you want, I can also show you 3 things that will make this assignment much easier:

- Exact Simulink model your professor expects (very typical robotics lab model)

- How to calculate yaw angle drift correctly

- How to finish the whole assignment in ~30 minutes in MATLAB

Just tell me and I’ll walk you through it step-by-step.

sure

Perfect! Let’s set you up to finish the whole experiment efficiently in MATLAB + Simulink. We’ll do it step by step, including everything from importing the data to producing the final plots.

# Step 1: Prepare the data in MATLAB

- Make sure you have gyro data and TopCon data as CSV files.
Example CSV format:

time,yaw_rate
0.00,0.12
0.01,0.15
0.02,0.14

- Load both datasets into MATLAB and create timeseries objects for Simulink:

% Load Gyro data
gyro = readtable('gyro_data.csv');
time_gyro = gyro.time;
yaw_rate_gyro = gyro.yaw_rate;

gyro_ts = timeseries(yaw_rate_gyro, time_gyro);

% Load TopCon ground truth
topcon = readtable('topcon_data.csv');
time_topcon = topcon.time;
yaw_rate_topcon = topcon.yaw_rate;

% Resample TopCon to gyro time if needed
yaw_rate_topcon_interp = interp1(time_topcon, yaw_rate_topcon, time_gyro);
topcon_ts = timeseries(yaw_rate_topcon_interp, time_gyro);

✅ Tip: Using interp1 ensures both signals align perfectly in time for comparison.

# Step 2: Build the Simulink model

### Blocks you need:

| Block | Function |
| --- | --- |
| From Workspace (gyro_ts) | Inputs gyro yaw rate |
| Integrator | Integrates yaw rate → yaw angle |
| From Workspace (topcon_ts) | Inputs TopCon yaw rate |
| Subtract | Computes yaw rate error |
| Scope | View signals in real-time |
| To Workspace | Save yaw angle and error to MATLAB |

### Simulink Layout

- Gyro yaw rate → Integrator → Yaw angle → Scope + To Workspace

- Gyro yaw rate → Subtract (with TopCon yaw rate) → Error → Scope + To Workspace

Logical flow:

Gyro yaw rate ---> Integrator ---> Yaw angle ---> Scope / To Workspace
        \
         \
          ---> Subtract <--- TopCon yaw rate ---> Error ---> Scope / To Workspace

# Step 3: Configure Integrator

- Initial condition = 0 (robot starts facing straight)

- Output = yaw angle in degrees (or radians depending on your gyro)

# Step 4: Run the simulation

- Set Stop time = max(time_gyro)

- Run the simulation

- Outputs to MATLAB workspace: yaw_angle_sim, error_sim

# Step 5: Plot results in MATLAB

### 1️⃣ Yaw rate comparison

figure;
plot(time_gyro, yaw_rate_gyro, 'b', 'LineWidth', 1.5); hold on;
plot(time_gyro, yaw_rate_topcon_interp, 'r--', 'LineWidth', 1.5);
xlabel('Time (s)'); ylabel('Yaw Rate (deg/s)');
title('Yaw Rate Comparison');
legend('Gyro','TopCon');
grid on;

### 2️⃣ Yaw angle from integration

figure;
plot(yaw_angle_sim.time, yaw_angle_sim.data, 'LineWidth', 1.5);
xlabel('Time (s)'); ylabel('Yaw Angle (deg)');
title('Yaw Angle from Gyro Integration');
grid on;

### 3️⃣ Error

error = yaw_rate_gyro - yaw_rate_topcon_interp;

figure;
plot(time_gyro, error, 'LineWidth', 1.5);
xlabel('Time (s)'); ylabel('Yaw Rate Error (deg/s)');
title('Gyro vs TopCon Error');
grid on;

% Compute RMSE
rmse = sqrt(mean(error.^2));
disp(['RMSE = ', num2str(rmse), ' deg/s']);

# Step 6: Speed test (optional)

If you want to test maximum speed:

- Gradually increase AlphaBot motor PWM in your control script.

- Record yaw rate and camera-based lane tracking.

- Plot tracking stability vs speed to find max stable speed.

# Step 7: Optional improvements

- Drift compensation: Use a complementary filter with camera yaw for long runs.

- Automatic simulation: Script to loop through multiple speed settings and save all plots automatically.

- Report-ready figures: Use set(gca,'FontSize',14) for clearer plots.

If you want, I can create a ready-to-use Simulink .slx file layout for you that matches this workflow exactly — you just need to load your CSV files and hit Run.

Do you want me to do that next?