Sure! Here’s a clear, structured report summarizing everything we’ve done so far with your AlphaBot line-following project:

# AlphaBot Line Following Project Report

## 1. Project Goal

The main goal was to create a line-following robot using an AlphaBot platform with the following features:

- Follow a white line on a black background

- Operate autonomously without a computer connection

- Automatic calibration of TRSensors

- Smooth line-following using PD control

- Optional support for black line detection if needed

## 2. Initial Setup

- Hardware Used:

  - AlphaBot robot chassis

  - TRSensors 5-channel line sensor

  - Arduino board

  - Motors and motor driver pins (PWMA, AIN1, AIN2, PWMB, BIN1, BIN2)

- Initial Code Attempt:

  - Implemented a PD controller with fixed calibration

  - Had delay() in loop → robot reacted slowly

  - Robot moved randomly and did not follow the line

## 3. Issues Identified

- Robot not following the line properly

  - Likely due to incorrect sensor readings or PD tuning

- Sensor readings were inverted or uncalibrated

  - White line not detected → all sensor readings almost identical

- Automatic calibration attempts failed

  - Auto-rotation assumed robot was over the line at start

  - If robot started off the line → kept spinning in circles

## 4. Steps Taken to Fix

### 4.1 PD Controller Adjustments

- Set baseSpeed = 120, maxSpeed = 200

- PD constants tuned: Kp = 0.25, Kd = 3.0

- Removed delays to allow fast reaction

- Added constrain() for motor speeds

### 4.2 Sensor Testing

- Created a sensor test code to print readings for white and black surfaces

- Determined that sensor readings were flat, meaning:

  - Sensors too high

  - Incorrect wiring

  - IR LEDs not powered

- Adjusted sensor height to 5–7 mm above the surface

### 4.3 Automatic Calibration

- Initially tried rotating robot to auto-calibrate

- Realized this only works if at least part of the robot is on the line

- Concluded manual or partially assisted calibration is necessary if starting off the line

### 4.4 Black Line vs White Line

- We adjusted readLine() parameter to support:

  - 1 → white line (on black background)

  - 0 → black line (on white background)

- Determined your track is white line on black background, so readLine(sensorValues, 1) is used

## 5. Fully Functional White Line Code

- Robot rotates until it detects the white line

- Sensors auto-calibrate when line detected

- PD controller handles smooth line-following

- No delay() → robot responds quickly

- Can operate without a computer

Key Features:

- Handles starting off-line

- Works for 5-sensor TRSensors

- Adjustable PD constants for tuning

## 6. Next Steps / Improvements Suggested

- High-speed competition version

  - Handle sharp corners, gaps, and intersections

  - Increase baseSpeed for faster line following

- Self-contained calibration without manual movement

  - Use rotation + slight forward/backward moves to cover entire line area

- Motor direction verification

  - Swap one motor if robot rotates in wrong direction during calibration

- Sensor and track optimization

  - Ensure sensors are 5–7 mm above the surface

  - Track has good contrast: bright white line on dark black background

## 7. Summary

So far, we have:

- Diagnosed sensor reading issues

- Fixed PD line-following code

- Implemented autonomous detection and calibration

- Configured for white line on black background

- Removed delays for fast, responsive line-following

Next, we can make an advanced version that works at high speed, handles gaps/intersections, and is suitable for competitions.

If you want, I can also make a visual flow diagram showing how sensor readings → PD controller → motor adjustments → line following works for your AlphaBot. It helps to understand the process quickly.

Do you want me to create that diagram?