Absolutely! Let’s summarize everything we’ve done so far with your robot project — step by step.

## 1. Original Goal

- Build a line-following robot using IR sensors.

- Robot should follow a black background with white lines, and stop when it sees a white line.

## 2. Initial Problems

- Original code had issues:

  - Motors were running backward or in opposite directions.

  - TRSensors library calls like read() or init() were not available in your version.

  - PID loop existed, but motor wiring and sensor calibration were incorrect.

## 3. Sensor Calibration

- Used TRSensors library.

- Calibration steps:

  - Call trs.calibrate() multiple times (400) in setup() while moving the robot over black and white.

  - Stores min and max values for each sensor.

  - Ensures accurate weighted line position with readLine().

- Corrected code uses:

trs.calibrate();    // calibrate each sensor
trs.AnalogRead(sensorValues); // read raw values
unsigned int position = trs.readLine(sensorValues); // get line position

## 4. Motor Control Issues

- Left/right motors sometimes ran in wrong directions.

- Fixed by swapping HIGH/LOW for one or both motors depending on wiring.

- Final configuration ensures both motors move forward.

Example:

// Left motor
digitalWrite(AIN1, HIGH);
digitalWrite(AIN2, LOW);

// Right motor
digitalWrite(BIN1, HIGH);
digitalWrite(BIN2, LOW);

## 5. PID Line Following

- Implemented PID control:

  - Kp, Ki, Kd constants adjust response.

  - PID calculates correction value based on line position deviation.

  - Correction is applied to left and right motor speeds.

- Constrained motor speeds to avoid negative values and backward movement.

## 6. Reverse Line Detection (White on Black)

- Original code assumed black line on white floor.

- Task: follow black background, stop on white line.

- Solution:

  - Check max sensor value each loop.

  - If it exceeds a threshold (WHITE_THRESHOLD), stop motors immediately.

if (maxSensor >= WHITE_THRESHOLD) {
  analogWrite(PWMA, 0);
  analogWrite(PWMB, 0);
  while(1); // stop forever
}

## 7. Current Status

- Robot:

  - Follows black background using PID.

  - Stops automatically when white line is detected.

  - Sensor calibration is automatic at startup.

  - Motor directions are fixed so both move forward physically.

✅ Next possible improvements:

- Smooth stopping instead of abrupt stop.

- Serial debug output: PID correction, motor speed, and sensor values — helpful for tuning Kp, Ki, Kd.

- Fine-tune WHITE_THRESHOLD based on lighting and surface.