clc;

clear;

close all;

%% Connect to Arduino TCP Server

t = tcpclient("192.168.1.23", 5000);

%% Data storage

counter     = [];

yawRate     = [];

filteredYaw = [];

yawAngle    = [];

alpha = 0.1;

%% Connection timer

lastDataTime = tic;

figure;

disp("Starting data acquisition...");

%% Main loop

while true

try

%% Check for incoming data

if t.NumBytesAvailable > 0

% Read line

data = readline(t);

% Reset timeout timer

lastDataTime = tic;

% Parse data

vals = str2double(split(strtrim(data), ","));

% Validate packet

if length(vals) < 3 || any(isnan(vals))

continue;

end

rawYaw = vals(1);

angle  = vals(2);

count  = vals(3);

%% Low-pass filter

if isempty(filteredYaw)

filtYaw = rawYaw;

else

filtYaw = (1 - alpha) * filteredYaw(end) + alpha * rawYaw;

end

%% Store data

yawRate(end+1)     = rawYaw;

filteredYaw(end+1) = filtYaw;

yawAngle(end+1)    = angle;

counter(end+1)     = count;

%% Live plotting

subplot(4,1,1)

plot(counter, yawRate, 'b', 'LineWidth', 1.5)

title('Raw Yaw Rate')

ylabel('deg/s')

grid on

subplot(4,1,2)

plot(counter, filteredYaw, 'g', 'LineWidth', 1.5)

title('Filtered Yaw Rate')

ylabel('deg/s')

grid on

subplot(4,1,3)

plot(counter, yawAngle, 'r', 'LineWidth', 1.5)

title('Yaw Angle')

ylabel('deg')

grid on

subplot(4,1,4)

plot(1:length(counter), counter, 'k', 'LineWidth', 1.5)

title('Counter')

ylabel('Count')

xlabel('Sample')

grid on

drawnow;

else

pause(0.01);

%% Stop if no data for 2 seconds

if toc(lastDataTime) > 2

disp("Robot disconnected or no data received.");

break;

end

end

catch ME

disp("Connection error occurred:");

disp(ME.message);

break;

end

end

%% Cleanup TCP connection

clear t;

%% Save data to file

save('yaw_data2.mat', 'counter', 'yawRate', 'filteredYaw', 'yawAngle');

disp("Data saved to yaw_data.mat");

disp("Script finished.");