Gyro Data

%**************************************************************************

%                   Hochschule Hamm-Lippstadt                             *

%**************************************************************************

% Modul	          : TopCon_vs_Gyro.m                                      *

%                                                                         *

% Date            : 10.06.2026                                            *

%                                                                         *

% Function        :  Getting Gyro Gy-85 sensor data via WLAN              *

%                     YawRate and YawAngle                                *

%                                                                         *

% Implementation  : R2025b                                                *

%                                                                         *

%                                                                         *

% Author          : Syed Muhammad Abis Rizvi                              *

% Reference       : AI (conceptual assistance and code structuring)       *

% Note                                                                    *

%                                                                         *

% Last Modified   : 18.06.2026                                            *

%                                                                         *

%**************************************************************************

clc;

clear;

close all;

%% Connect to Arduino TCP Server

t = tcpclient("192.168.1.23", 5000);

%% Data storage

counter     = [];

yawRate     = [];

filteredYaw = [];

yawAngle    = [];

packetTime  = [];

alpha = 0.1; %% Sets the smoothing factor%%

%% Timers

lastDataTime = tic;

startTime    = tic;

%% Autosave settings

autosaveFile = 'yaw_data_autosave.mat';

autosaveEverySamples = 100;

%% Create figure

hFig = figure;

disp("Starting data acquisition...");

%% Main loop

while true

try

%% Stop and final save if figure is closed

if ~isvalid(hFig)

saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, "final");

break;

end

%% Check for incoming data

if t.NumBytesAvailable > 0

%% Read line from Arduino

data = readline(t);

currentPacketTime = toc(startTime);

disp("Received:")

disp(data)

%% Reset timeout timer

lastDataTime = tic;

%% Parse data

vals = str2double(split(strtrim(data), ","));

disp("Parsed values:")

disp(vals)

%% Arduino sends: yawRate, yawAngle, counter

if numel(vals) ~= 3 || any(isnan(vals))

disp("Invalid packet:")

disp(data)

continue;

end

rawYaw = vals(1);

angle  = vals(2);

count  = vals(3);

%% Low-pass filter in MATLAB

if isempty(filteredYaw)

filtYaw = rawYaw;

else

filtYaw = (1 - alpha) * filteredYaw(end) + alpha * rawYaw;

end

%% Store data

yawRate(end+1)     = rawYaw;

filteredYaw(end+1) = filtYaw;

yawAngle(end+1)    = angle;

counter(end+1)     = count;

packetTime(end+1)  = currentPacketTime;

%% Autosave while running

if mod(length(counter), autosaveEverySamples) == 0

save(autosaveFile, ...

'counter', ...

'yawRate', ...

'filteredYaw', ...

'yawAngle', ...

'packetTime');

disp(['Autosaved to ', autosaveFile]);

end

%% Plot using MATLAB receive timestamp

plotTime = packetTime - packetTime(1);

subplot(4,1,1)

plot(plotTime, yawRate, 'b', 'LineWidth', 1.5)

title('Raw Yaw Rate')

ylabel('deg/s')

grid on

subplot(4,1,2)

plot(plotTime, filteredYaw, 'g', 'LineWidth', 1.5)

title('Filtered Yaw Rate')

ylabel('deg/s')

grid on

subplot(4,1,3)

plot(plotTime, yawAngle, 'r', 'LineWidth', 1.5)

title('Yaw Angle')

ylabel('deg')

grid on

subplot(4,1,4)

plot(plotTime, counter, 'k', 'LineWidth', 1.5)

title('Counter')

ylabel('Count')

xlabel('Time (s)')

grid on

drawnow;

else

pause(0.01);

%% Stop and save only if no data for 20 seconds

if toc(lastDataTime) > 20

disp("Robot disconnected or no data received.");

saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, "final");

break;

end

end

catch ME

disp("Connection error occurred:");

disp(ME.message);

saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, "final");

break;

end

end

%% Cleanup TCP connection

clear t;

disp("Script finished.");

%% Function to save yaw data

function saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, mode)

if isempty(counter)

disp("No valid data received. Nothing useful to save.");

return;

end

timestamp = datestr(now, 'yyyymmdd_HHMMSS');

if mode == "final"

filename = sprintf('yaw_data_%s.mat', timestamp);

else

filename = 'yaw_data_autosave.mat';

end

save(filename, ...

'counter', ...

'yawRate', ...

'filteredYaw', ...

'yawAngle', ...

'packetTime');

disp(['Data saved as: ', filename]);

end

Look for gyro data

%**************************************************************************

%                   Hochschule Hamm-Lippstadt                             *

%**************************************************************************

% Modul	          : TopCon_vs_Gyro.m                                      *

%                                                                         *

% Date            : 10.06.2026                                            *

%                                                                         *

% Function        :  Getting Gyro Gy-85 sensor data via WLAN              *

%                     YawRate and YawAngle                                *

%                                                                         *

% Implementation  : R2025b                                                *

%                                                                         *

%                                                                         *

% Author          : Syed Muhammad Abis Rizvi                              *

% Reference       : AI (conceptual assistance and code structuring)       *

% Note                                                                    *

%                                                                         *

% Last Modified   : 18.06.2026                                            *

%                                                                         *

%**************************************************************************

clear; clc; close all;

%% Load gyro file

S = load('yaw_data_20260615_115641.mat');

%% Extract data

yawRate = S.filteredYaw;

yawAngle = S.yawAngle;

%% Time axis

if isfield(S,'packetTime')

t = S.packetTime;

elseif isfield(S,'arduinoTime')

t = S.arduinoTime;

else

t = S.counter * 0.02;   % fallback

end

t = t - t(1);

%% Plot yaw rate

figure;

plot(t, yawRate,'b','LineWidth',1.5);

grid on;

xlabel('Time (s)');

ylabel('Yaw Rate (deg/s)');

title('Gyro Yaw Rate');

%% Plot yaw angle

figure;

plot(t, yawAngle,'r','LineWidth',1.5);

grid on;

xlabel('Time (s)');

ylabel('Yaw Angle (deg)');

title('Gyro Yaw Angle');

%% Show duration

fprintf('Duration = %.2f seconds\n', t(end));

fprintf('Samples  = %d\n', length(t));

MAPPING

clear all; close all; clc;

KalmanFilter(); % Reset Kalman-Filter

% Daten aus dem Datenordner laden

stAktuellerOrdner = cd;

stDatenOrdner = [stAktuellerOrdner(1:findstr(stAktuellerOrdner,'m-files')-1),'Daten'];

addpath(stDatenOrdner);

%% Karte laden

load('Rundkurs.mat');

%% Daten laden

cd(stDatenOrdner)

[stDateiName,~] = uigetfile("*.mat","Wähle Messung Messung_*.mat");

load("Messung_20260615_115841.mat");

disp([stDateiName,' wurde geladen.'])

cd(stAktuellerOrdner)

%% Grafik vorbereiten

figure

hold on

% Rundkurs

h1 = plot(LinkeFahrspur_W(1,:),LinkeFahrspur_W(2,:),'k-');

h2 = plot(RechteFahrspur_W(1,:),RechteFahrspur_W(2,:),'k-');

h3 = plot(MittelLinie_W(1,:),MittelLinie_W(2,:),'k--');

plot(Messung(1,:),Messung(2,:),'r-')

axis equal

xlabel('x_W in m')

ylabel('y_W in m')

title(stDateiName,'Interpreter','none')

% Roboter Pose zeichnen

MeinRoboter.x = 2;

MeinRoboter.y = 0.2;

MeinRoboter.Kurs = 0.0;

hRoboter     = plot(MeinRoboter.x,MeinRoboter.y,'bo','MarkerSize',10); % Wahrer Wert des Roboters

fKursLinienLaenge = 0.5;

hRoboterKurs = line([MeinRoboter.x, MeinRoboter.x+fKursLinienLaenge*cosd(MeinRoboter.Kurs)],[MeinRoboter.y, MeinRoboter.y+fKursLinienLaenge*sind(MeinRoboter.Kurs)],'Color','b'); % Roboter

%% Zyklusschleife

t1 = 0; % Zeitmessung initialisieren

tic;

for i=1:length(xEst)

%

if i==1

dt = 0.15;

else

dt = Zeitstempel(i)-Zeitstempel(i-1);

end

xEst(:,i)=KalmanFilter([Messung(1,i);Messung(2,i)],dt); % [x;y;vx;vy]

% Kursschätzung AP 2.6

Kurs=atan2d(xEst(4,i),xEst(3,i));

RoboterPose(:,i) = [xEst(1,i);xEst(2,i);Kurs];

% Karte aktualisieren

zeichenRoboter(hRoboter,hRoboterKurs,RoboterPose, i);

%pause(0.1)

end

%% Darstellung der Zustandsschätzung

figure

subplot 411

plot(Zeitstempel,Messung(1,:),'r.-')

hold on

plot(Zeitstempel,xEst(1,:),'b.-')

xlabel('Zeit in s')

ylabel('x in m')

legend('Messung','Kalman-Filter','Location','southeast')

subplot 412

plot(Zeitstempel,Messung(2,:),'r.-')

hold on

plot(Zeitstempel,xEst(2,:),'b.-')

xlabel('Zeit in s')

ylabel('y in m')

legend('Messung','Kalman-Filter','Location','southeast')

subplot 413

plot(Zeitstempel,xEst(3,:),'b.-')

xlabel('Zeit in s')

ylabel('v_x in m/s')

subplot 414

plot(Zeitstempel,xEst(4,:),'b.-')

xlabel('Zeit in s')

ylabel('v_y in m/s')

%% Zeichne Pose in Karte AP 2.7

function zeichenRoboter(hRoboter,hRoboterKurs,Pose,i)

% Aktualisiere die Roboterposition in der Grafik

fKursLinienLaenge = 0.2;

set(hRoboter, 'XData', Pose(1,i), 'YData', Pose(2,i));

set(hRoboterKurs, 'XData', [Pose(1,i), Pose(1,i) + fKursLinienLaenge *cosd(Pose(3,i))], ...

'YData', [Pose(2,i), Pose(2,i) + fKursLinienLaenge *sind(Pose(3,i))]);

plot(Pose(1,:),Pose(2,:),'b-')

drawnow; % Aktualisiere die Grafik

end

Comparison

%**************************************************************************

%                   Hochschule Hamm-Lippstadt                             *

%**************************************************************************

% Modul	          : TopCon_vs_Gyro.m                                      *

%                                                                         *

% Date            : 10.06.2026                                            *

%                                                                         *

% Function        : Comparing TopCon and Gyro Gy-85 sensor data           *

%                                                                         *

%                                                                         *

% Implementation  : R2025b                                                *

%                                                                         *

%                                                                         *

% Author          : Syed Muhammad Abis Rizvi                              *

% Reference       : AI (conceptual assistance and code structuring)       *

% Note              MATLAB files provided by Prof Schneider contained

%                   infromation about track and were used to  record data

%                   from TopCon.                                          *

%                                                                         *

% Last Modified : 18.06.2026                                              *

%                                                                         *

%**************************************************************************

clear; clc; close all;

%% ===================== DATA PATH =====================

dataPath = 'D:\SVN\AlphaBot\Syed\Matlab Codes In English';

gyroFile   = fullfile(dataPath,'yaw_data_20260615_115641.mat');

topconFile = fullfile(dataPath,'Messung_20260615_115817.mat');

mapFile    = fullfile(dataPath,'Rundkurs.mat');

%% ===================== LOAD GYRO DATA =====================

S = load(gyroFile);

counter     = S.counter(:);

yawRate_deg = S.filteredYaw(:);

yaw_deg_raw = S.yawAngle(:);

%% ===================== GYRO TIME VECTOR =====================

if isfield(S,'packetTime') && ~isempty(S.packetTime)

t = S.packetTime(:);

elseif isfield(S,'arduinoTime') && ~isempty(S.arduinoTime)

t = S.arduinoTime(:);

else

t = counter * 0.02;     % fallback for 50 Hz

end

t = t - t(1);

%% ===================== CLEAN GYRO DATA =====================

valid = isfinite(t) & isfinite(yaw_deg_raw) & isfinite(yawRate_deg);

t           = t(valid);

yaw_deg_raw = yaw_deg_raw(valid);

yawRate_deg = yawRate_deg(valid);

yaw_deg_raw = yaw_deg_raw - yaw_deg_raw(1);

%% ===================== LOAD TOPCON DATA =====================

if ~isfile(topconFile)

error('Topcon file not found.');

end

load(topconFile);   % loads Pose, Zeitstempel, xEst

t_gt = Zeitstempel(:);

t_gt = t_gt - t_gt(1);

%% ===================== TOPCON YAW =====================

% IMPORTANT:

% No minus sign here. Your plot showed the Topcon sign was inverted.

yaw_gt_deg = Pose(3,:)';

yaw_gt_deg = unwrap(deg2rad(yaw_gt_deg));

yaw_gt_deg = rad2deg(yaw_gt_deg);

yaw_gt_deg = yaw_gt_deg - yaw_gt_deg(1);

%% ===================== TOPCON YAW RATE =====================

yawRate_gt = zeros(size(yaw_gt_deg));

for k = 2:length(yaw_gt_deg)

dt_gt = t_gt(k) - t_gt(k-1);

if dt_gt > 0

yawRate_gt(k) = (yaw_gt_deg(k) - yaw_gt_deg(k-1)) / dt_gt;

end

end

yawRate_gt = movmean(yawRate_gt, 5);

%% ===================== AUTOMATIC TIME SYNCHRONIZATION =====================

gyroThreshold   = 5;

topconThreshold = 5;

idxGyroStart = find(abs(yawRate_deg) > gyroThreshold, 1, 'first');

idxTopStart  = find(abs(yawRate_gt)  > topconThreshold, 1, 'first');

if isempty(idxGyroStart)

idxGyroStart = 1;

end

if isempty(idxTopStart)

idxTopStart = 1;

end

gyroStartTime   = t(idxGyroStart);

topconStartTime = t_gt(idxTopStart);

timeShift = topconStartTime - gyroStartTime - 2;

t_gt_sync = t_gt - timeShift;

fprintf('Automatic time shift applied: %.3f s\n', timeShift);

%% ===================== AUTOMATIC GYRO SCALE =====================

% Compare only overlapping time range

tMin = max(min(t), min(t_gt_sync));

tMax = min(max(t), max(t_gt_sync));

idxG = t >= tMin & t <= tMax;

idxT = t_gt_sync >= tMin & t_gt_sync <= tMax;

gyroMax   = max(abs(yaw_deg_raw(idxG)));

topconMax = max(abs(yaw_gt_deg(idxT)));

if gyroMax > 0

gyroScale = 0.65;

else

gyroScale = 1.0;

end

fprintf('Automatic gyro scale: %.4f\n', gyroScale);

yaw_deg     = yaw_deg_raw * gyroScale;

yawRate_deg = yawRate_deg * gyroScale;

yaw = deg2rad(yaw_deg);

%% ===================== YAW RATE COMPARISON =====================

figure('Name','Yaw Rate Comparison','Color','w');

plot(t, yawRate_deg, 'b','LineWidth',1.3); hold on;

plot(t_gt_sync, yawRate_gt, 'r','LineWidth',1.3);

grid on;

xlabel('Time (s)');

ylabel('Yaw Rate (deg/s)');

title('Yaw Rate Comparison');

legend('Gyro','Topcon');

%% =====================  YAW ANGLE COMPARISON =====================

figure('Name',' Yaw Angle Comparison','Color','w');

plot(t, yaw_deg, 'b','LineWidth',1.3); hold on;

plot(t_gt_sync, yaw_gt_deg, 'r','LineWidth',1.3);

grid on;

xlabel('Time (s)');

ylabel('Yaw Angle (deg)');

title(' Yaw Angle Comparison');

legend('Gyro','Topcon');

hold off;

Control

/*******************************************************************************************************

Hochschule Hamm-Lippstadt

*******************************************************************************************************

Modul           :   AlphaBot

Date            :   26.05.2026

Function        :   Lane detection with pixy2 camera and recording yaw rate and yaw angle.

Hardware        :   UNO R4 WIFI, Pixy2 camera and Gy-85 sensor

Implementation  :   Arduino IDE 2.3.86

Author          :   Syed Muhammad Abis Rizvi

References       :   Waveshare AlphaBot demo code (www.waveshare.com/wiki/AlphaBot)

GY-85 IMU sensor  Arduino examples and documentation (https://docs.arduino.cc/)

Pixy2 examples and documentation (https://docs.pixycam.com/wiki/doku.php?id=wiki:v2:arduino_api)

AI (conceptual assistance and code structuring)

Last Modified   :    12.06.2026

*******************************************************************************************************/

#include <WiFiS3.h>

#include <Pixy2I2C.h>

#include <Wire.h>

// WiFi Credentials

char ssid[] = "AutonomeSysteme";

char pass[] = "Kennwort1";

// Networking Definitions

IPAddress local_IP(192, 168, 1, 23);

IPAddress gateway(192, 168, 1, 1);

IPAddress subnet(255, 255, 255, 0);

WiFiServer server(5000);

WiFiClient matlabClient;

// -------------------- PIXY --------------------

Pixy2I2C pixy;

// -------------------- GYRO --------------------

#define ITG3205_ADDR 0x68

// -------------------- MOTOR PINS --------------------

#define ENA 5

#define ENB 6

#define IN1 A1

#define IN2 A0

#define IN3 A2

#define IN4 A3

// -------------------- PID CONSTANTS (TUNED FOR MAX SPEED) --------------------

float Kp = 7.6;   // Increased to react fast enough at maximum speed (without 6.0/PRISMA 6.6 )

float Kd = 4.0;   // Increased to prevent extreme high-speed weaving (without 3.8/PRISMA 4.0 )

float Ky = 0.06;  // Increased to lock straight lines during full throttle (without 0.06/PRISMA 0.06 )

float Kpsi = 0.04;

// ===================== SETTINGS =====================

const int frameCenter = 39;

const int laneShift   = 20; // horizontal pixel offset use to calculate a virtual center line (without 22/PRISMA 20)

// -------------------- MAXIMUM SPEED CONFIGURATION --------------------

const int maxStraightSpeed = 255; // Max speed PWM (Pulse Width Modulation)

const int minCornerSpeed   = 80;  // min speed during turns

// -------------------- STATE & MEMORY --------------------

int lastError = 0;

float filteredGyro = 0;

float gyroOffsetZ = 0;

float lastValidSteering = 0;

int lostFrameCounter = 0;

float yawAngle = 0;

float headingError = 0;

float integralError = 0;

float lineHeading = 0;

unsigned long lastMicros = 0;

unsigned long lastWiFiMillis = 0; //

unsigned long sendCounter = 0;    //

// Smooth Steering Memory

float smoothedSteering = 0;

const float steeringFilter = 0.45; // Faster filtering to allow quick high-speed turns

// -------------------- GYRO READ --------------------

int16_t readRawGyroZ() {

Wire.beginTransmission(ITG3205_ADDR);

Wire.write(0x21);

Wire.endTransmission(false);

Wire.requestFrom(ITG3205_ADDR, 2);

if (Wire.available() >= 2) { // Requests 2 bytes of data (High byte and Low byte) from the gyro.

return (Wire.read() << 8) | Wire.read(); // Combines the two 8-bit readings into a single signed 16-bit integer via bit-shifting (<< 8) and bitwise

}

return 0;

}

// -------------------- MOTOR CONTROL --------------------

void setMotor(int left, int right) {

// Constrained precisely to the Arduino hardware limit of 255

left = constrain(left, -255, 255);

right = constrain(right, -255, 255);

digitalWrite(IN1, left >= 0 ? HIGH : LOW);

digitalWrite(IN2, left >= 0 ? LOW : HIGH);

digitalWrite(IN3, right >= 0 ? HIGH : LOW);

digitalWrite(IN4, right >= 0 ? LOW : HIGH);

analogWrite(ENA, abs(left));

analogWrite(ENB, abs(right));

}

// -------------------- SETUP --------------------

void setup() {

Serial.begin(115200);

Wire.begin();

pixy.init();

pixy.changeProg("line");

pinMode(ENA, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

Wire.beginTransmission(ITG3205_ADDR);

Wire.write(0x3E);

Wire.write(0x00);

Wire.endTransmission();

Serial.println("Calibrating Gyro...");

long sumZ = 0;

for (int i = 0; i < 1000; i++) {

sumZ += readRawGyroZ();

delay(2);

}

gyroOffsetZ = sumZ / 1000.0;

Serial.println("Gyro Ready");

// -------------------- WLAN INITIALIZATION --------------------

// Configured inside setup() to prevent code blocking loops later

WiFi.config(local_IP, gateway, subnet);

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

server.begin();

lastMicros = micros();

lastWiFiMillis = millis();

Serial.println("\nWireless TCP Server Ready at 192.168.1.23:5000");

}

// -------------------- MAIN LOOP --------------------

void loop() {

// -------------------- TIME DELTA CALCULATION --------------------

unsigned long currentMicros = micros();

float dt = (currentMicros - lastMicros) / 1000000.0; // Convert to seconds

lastMicros = currentMicros;

// -------------------- GYRO PROCESSING --------------------

float yawRate = (readRawGyroZ() - gyroOffsetZ) / 14.375;

if (abs(yawRate) < 1.0) {

yawRate = 0;

}

filteredGyro = 0.9 * filteredGyro + 0.1 * yawRate; // Applies a Low-Pass filter (90% historical data, 10% new data) to eliminate vibration noise

float gyroCorrection = filteredGyro * Ky; // steering correction

// -------------------- YAW ANLGE --------------------

yawAngle += filteredGyro * dt; // Integration calculation

// -------------------- MATLAB TCP SERVER --------------------

// Check for newly connecting clients

if (!matlabClient || !matlabClient.connected()) {

matlabClient = server.accept(); //  Client assignment for UNO R4 WiFi

}

// Send telemetry packets at 50Hz (every 20ms)

if (matlabClient && matlabClient.connected()) {

if (millis() - lastWiFiMillis >= 20) {

lastWiFiMillis = millis();

matlabClient.print(filteredGyro);

matlabClient.print(",");

matlabClient.print(yawAngle);

matlabClient.print(",");

matlabClient.println(sendCounter++);

}

}

// -------------------- PIXY FEATURE SELECTION --------------------

pixy.line.getMainFeatures();

int numVectors = pixy.line.numVectors;

if (numVectors > 0) {

lostFrameCounter = 0;

// -------------------- LINE HEADING --------------------

int x0 = pixy.line.vectors[0].m_x0;

int y0 = pixy.line.vectors[0].m_y0;

int x1 = pixy.line.vectors[0].m_x1;

int y1 = pixy.line.vectors[0].m_y1;

// CORRECT VECTOR DIRECTION IF PIXY FLIPS THE VECTOR-----------

if (y1 > y0) {

int tempX = x0;

x0 = x1;

x1 = tempX;

int tempY = y0;

y0 = y1;

y1 = tempY;

}

int dx = x1 - x0;

int dy = y1 - y0;

lineHeading = atan2(dy, dx) * 180.0 / PI;

// Desired straight-line heading in Pixy image coordinates

float desiredHeading = -133.0;

// -------------------- HEADING ERROR --------------------

headingError = lineHeading - desiredHeading;

// Normalize to -180 ... 180 degrees

if (headingError > 180) headingError -= 360;

if (headingError < -180) headingError += 360;

// -------------------- TARGET CENTER --------------------

int calculatedTargetX = frameCenter;

if (numVectors >= 2) {

int line1_X = (pixy.line.vectors[0].m_x0 + pixy.line.vectors[0].m_x1) / 2;

int line2_X = (pixy.line.vectors[1].m_x0 + pixy.line.vectors[1].m_x1) / 2;

calculatedTargetX = (line1_X + line2_X) / 2;  // find midpoint of detercted line which calculates a precise trajectory right through the middle of the path.

}

else {

int singleLineX = (pixy.line.vectors[0].m_x0 + pixy.line.vectors[0].m_x1) / 2; // One line detected: Finds the midpoint of the single line. If it is on the right side of the screen, it shifts the target left by laneShift pixels. If it is on the left, it shifts it right.

if (singleLineX > frameCenter) {

calculatedTargetX = singleLineX - laneShift;

} else {

calculatedTargetX = singleLineX + laneShift;

}

}

// -------------------- LANE OFFSET --------------------

int error = calculatedTargetX - frameCenter;

error = constrain(error, -28, 28); // spatial error distance which limits the error window to a max 28 pixels left or right

// -------------------- CURVATURE --------------------

// Derivative of lane error (rate of change of tracking error)

float curvature = (error - lastError) / dt;

lastError = error;

// -------------------- HEADING ERROR --------------------

// headingError = lineHeading - yawAngle;

// -------------------- SPEED CONTROL WITH EXPONENTIAL CORNER BRAKING --------------------

// Rapidly drops power when a turn is detected to keep the vehicle on the track

int currentBaseSpeed = maxStraightSpeed - (abs(error) * 5); //  lowers the robot's base speed by 5 units for every pixel of tracking error.

currentBaseSpeed = constrain(currentBaseSpeed, minCornerSpeed, maxStraightSpeed);

// -------------------- PID STEERING --------------------

float rawSteering =

(error * Kp) + (headingError * Kpsi) +(filteredGyro * Ky) +(curvature * Kd);

// Increased steering headroom up to 110 to force swift corrections at 255 top speed

rawSteering = constrain(rawSteering, -110, 110); // max range of 110 to avoid over-steering spinouts.

// Smooth sudden adjustments to maintain traction

smoothedSteering = (steeringFilter * rawSteering) + ((1.0 - steeringFilter) * smoothedSteering);// U

lastValidSteering = smoothedSteering;

Serial.println(smoothedSteering);

// -------------------- MOTOR DRIVE --------------------

setMotor(currentBaseSpeed - smoothedSteering, currentBaseSpeed + smoothedSteering);

} else {

// -------------------- MEMORY FALLBACK MODE --------------------

lostFrameCounter++;

if (lostFrameCounter < 15) {  // Shorter time frame because the robot covers more distance at high speeds

setMotor(minCornerSpeed - lastValidSteering, minCornerSpeed + lastValidSteering);

}

else {

setMotor(0, 0);

Serial.println("[CRITICAL LOST] Track Lost at Max Speed.");

}

}

delay(4); // Reduced  for faster execution cycles

}

LAST

#include <WiFiS3.h>

#include <Pixy2I2C.h>

#include <Wire.h>

// WiFi Credentials

char ssid[] = "AutonomeSysteme";

char pass[] = "Kennwort1";

// Networking Definitions

IPAddress local_IP(192, 168, 1, 23);

IPAddress gateway(192, 168, 1, 1);

IPAddress subnet(255, 255, 255, 0);

WiFiServer server(5000);

WiFiClient matlabClient;

// -------------------- PIXY --------------------

Pixy2I2C pixy;

// -------------------- GYRO --------------------

#define ITG3205_ADDR 0x68

// -------------------- MOTOR PINS --------------------

#define ENA 5

#define ENB 6

#define IN1 A1

#define IN2 A0

#define IN3 A2

#define IN4 A3

// -------------------- PID CONSTANTS (TUNED FOR MAX SPEED) --------------------

float Kp = 7.6;   // Increased to react fast enough at maximum speed (without 6.0/PRISMA 6.6 )

float Kd = 4.0;   // Increased to prevent extreme high-speed weaving (without 3.8/PRISMA 4.0 )

float Ky = 0.06;  // Increased to lock straight lines during full throttle (without 0.06/PRISMA 0.06 )

float Kpsi = 0.01;

// ===================== SETTINGS =====================

const int frameCenter = 39;

const int laneShift   = 20; // horizontal pixel offset use to calculate a virtual center line (without 22/PRISMA 20)

// -------------------- MAXIMUM SPEED CONFIGURATION --------------------

int maxStraightSpeed = 255; // Max speed PWM (Pulse Width Modulation)

int minCornerSpeed   = 80;  // min speed during turns

// -------------------- STATE & MEMORY --------------------

int lastError = 0;

float filteredGyro = 0;

float gyroOffsetZ = 0;

float lastValidSteering = 0;

int lostFrameCounter = 0;

float yawAngle = 0;

float headingError = 0;

float integralError = 0;

float lineHeading = 0;

unsigned long lastMicros = 0;

unsigned long lastWiFiMillis = 0; //

unsigned long sendCounter = 0;    //

// Smooth Steering Memory

float smoothedSteering = 0;

const float steeringFilter = 0.45; // Faster filtering to allow quick high-speed turns

// -------------------- GYRO READ --------------------

int16_t readRawGyroZ() {

Wire.beginTransmission(ITG3205_ADDR);

Wire.write(0x21);

Wire.endTransmission(false);

Wire.requestFrom(ITG3205_ADDR, 2);

if (Wire.available() >= 2) { // Requests 2 bytes of data (High byte and Low byte) from the gyro.

return (Wire.read() << 8) | Wire.read(); // Combines the two 8-bit readings into a single signed 16-bit integer via bit-shifting (<< 8) and bitwise

}

return 0;

}

// -------------------- MOTOR CONTROL --------------------

void setMotor(int left, int right) {

// Constrained precisely to the Arduino hardware limit of 255

left = constrain(left, -255, 255);

right = constrain(right, -255, 255);

digitalWrite(IN1, left >= 0 ? HIGH : LOW);

digitalWrite(IN2, left >= 0 ? LOW : HIGH);

digitalWrite(IN3, right >= 0 ? HIGH : LOW);

digitalWrite(IN4, right >= 0 ? LOW : HIGH);

analogWrite(ENA, abs(left));

analogWrite(ENB, abs(right));

}

// -------------------- SETUP --------------------

void setup() {

Serial.begin(115200);

Wire.begin();

pixy.init();

pixy.changeProg("line");

pinMode(ENA, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

Wire.beginTransmission(ITG3205_ADDR);

Wire.write(0x3E);

Wire.write(0x00);

Wire.endTransmission();

Serial.println("Calibrating Gyro...");

long sumZ = 0;

for (int i = 0; i < 1000; i++) {

sumZ += readRawGyroZ();

delay(2);

}

gyroOffsetZ = sumZ / 1000.0;

Serial.println("Gyro Ready");

// -------------------- WLAN INITIALIZATION --------------------

// Configured inside setup() to prevent code blocking loops later

WiFi.config(local_IP, gateway, subnet);

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

server.begin();

lastMicros = micros();

lastWiFiMillis = millis();

Serial.println("\nWireless TCP Server Ready at 192.168.1.23:5000");

}

// -------------------- MAIN LOOP --------------------

void loop() {

// -------------------- TIME DELTA CALCULATION --------------------

unsigned long currentMicros = micros();

float dt = (currentMicros - lastMicros) / 1000000.0; // Convert to seconds

lastMicros = currentMicros;

// -------------------- GYRO PROCESSING --------------------

float yawRate = (readRawGyroZ() - gyroOffsetZ) / 14.375;

if (abs(yawRate) < 1.0) {

yawRate = 0;

}

filteredGyro = 0.9 * filteredGyro + 0.1 * yawRate; // Applies a Low-Pass filter (90% historical data, 10% new data) to eliminate vibration noise

float gyroCorrection = filteredGyro * Ky; // steering correction

// -------------------- YAW ANLGE --------------------

yawAngle += filteredGyro * dt; // Integration calculation

// -------------------- MATLAB TCP SERVER --------------------

// Check for newly connecting clients

if (!matlabClient || !matlabClient.connected()) {

matlabClient = server.accept(); //  Client assignment for UNO R4 WiFi

}

// Send telemetry packets at 50Hz (every 20ms)

if (matlabClient && matlabClient.connected()) {

if (millis() - lastWiFiMillis >= 20) {

lastWiFiMillis = millis();

matlabClient.print(filteredGyro);

matlabClient.print(",");

matlabClient.print(yawAngle);

matlabClient.print(",");

matlabClient.println(sendCounter++);

}

}

// -------------------- PIXY FEATURE SELECTION --------------------

pixy.line.getMainFeatures();

int numVectors = pixy.line.numVectors;

if (numVectors > 0) {

lostFrameCounter = 0;

// -------------------- LINE HEADING --------------------

int x0 = pixy.line.vectors[0].m_x0;

int y0 = pixy.line.vectors[0].m_y0;

int x1 = pixy.line.vectors[0].m_x1;

int y1 = pixy.line.vectors[0].m_y1;

// Correct vector direction if Pixy flips the vector

if (y1 > y0) {

int tempX = x0;

x0 = x1;

x1 = tempX;

int tempY = y0;

y0 = y1;

y1 = tempY;

}

int dx = x1 - x0;

int dy = y1 - y0;

lineHeading = atan2(dy, dx) * 180.0 / PI;

// Desired straight-line heading in Pixy image coordinates

float desiredHeading = -90.0;

headingError = lineHeading - desiredHeading;

// Normalize to -180 ... 180 degrees

if (headingError > 180) headingError -= 360;

if (headingError < -180) headingError += 360;

// -------------------- TARGET CENTER --------------------

int calculatedTargetX = frameCenter;

if (numVectors >= 2) {

int line1_X = (pixy.line.vectors[0].m_x0 + pixy.line.vectors[0].m_x1) / 2;

int line2_X = (pixy.line.vectors[1].m_x0 + pixy.line.vectors[1].m_x1) / 2;

calculatedTargetX = (line1_X + line2_X) / 2;  // find midpoint of detercted line which calculates a precise trajectory right through the middle of the path.

}

else {

int singleLineX = (pixy.line.vectors[0].m_x0 + pixy.line.vectors[0].m_x1) / 2; // One line detected: Finds the midpoint of the single line. If it is on the right side of the screen, it shifts the target left by laneShift pixels. If it is on the left, it shifts it right.

if (singleLineX > frameCenter) {

calculatedTargetX = singleLineX - laneShift;

} else {

calculatedTargetX = singleLineX + laneShift;

}

}

// -------------------- LANE OFFSET --------------------

int error = calculatedTargetX - frameCenter;

error = constrain(error, -28, 28); // spatial error distance which limits the error window to a max 28 pixels left or right

// -------------------- CURVATURE --------------------

int curvature = error - lastError;

lastError = error;

// -------------------- HEADING ERROR --------------------

//headingError = lineHeading - yawAngle;

// -------------------- SPEED CONTROL WITH EXPONENTIAL CORNER BRAKING --------------------

// Rapidly drops power when a turn is detected to keep the vehicle on the track

int currentBaseSpeed = maxStraightSpeed - (abs(error) * 5); //  lowers the robot's base speed by 5 units for every pixel of tracking error.

currentBaseSpeed = constrain(currentBaseSpeed, minCornerSpeed, maxStraightSpeed);

// -------------------- PID STEERING --------------------

float rawSteering =

(error * Kp) + (headingError * Kpsi) +(filteredGyro * Ky) +(curvature * Kd);

// Increased steering headroom up to 110 to force swift corrections at 255 top speed

rawSteering = constrain(rawSteering, -110, 110); // max range of 110 to avoid over-steering spinouts.

// Smooth sudden adjustments to maintain traction

smoothedSteering = (steeringFilter * rawSteering) + ((1.0 - steeringFilter) * smoothedSteering);

lastValidSteering = smoothedSteering;

// -------------------- MOTOR DRIVE --------------------

setMotor(currentBaseSpeed - smoothedSteering, currentBaseSpeed + smoothedSteering);

} else {

// -------------------- MEMORY FALLBACK MODE --------------------

lostFrameCounter++;

if (lostFrameCounter < 15) {  // Shorter time frame because the robot covers more distance at high speeds

setMotor(minCornerSpeed - lastValidSteering, minCornerSpeed + lastValidSteering);

}

else {

setMotor(0, 0);

Serial.println("[CRITICAL LOST] Track Lost at Max Speed.");

}

}

delay(4); // Reduced  for faster execution cycles

}