clc;

clear;

close all;

%% Connect to Arduino TCP Server

t = tcpclient("192.168.1.23", 5000);

%% Data storage

counter     = [];

yawRate     = [];

filteredYaw = [];

yawAngle    = [];

packetTime  = [];

alpha = 0.1; %% Sets the smoothing factor%%

%% Timers

lastDataTime = tic;

startTime    = tic;

%% Autosave settings

autosaveFile = 'yaw_data_autosave.mat';

autosaveEverySamples = 100;

%% Create figure

hFig = figure;

disp("Starting data acquisition...");

%% Main loop

while true

try

%% Stop and final save if figure is closed

if ~isvalid(hFig)

saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, "final");

break;

end

%% Check for incoming data

if t.NumBytesAvailable > 0

%% Read line from Arduino

data = readline(t);

currentPacketTime = toc(startTime);

disp("Received:")

disp(data)

%% Reset timeout timer

lastDataTime = tic;

%% Parse data

vals = str2double(split(strtrim(data), ","));

disp("Parsed values:")

disp(vals)

%% Arduino sends: yawRate, yawAngle, counter

if numel(vals) ~= 3 || any(isnan(vals))

disp("Invalid packet:")

disp(data)

continue;

end

rawYaw = vals(1);

angle  = vals(2);

count  = vals(3);

%% Low-pass filter in MATLAB

if isempty(filteredYaw)

filtYaw = rawYaw;

else

filtYaw = (1 - alpha) * filteredYaw(end) + alpha * rawYaw;

end

%% Store data

yawRate(end+1)     = rawYaw;

filteredYaw(end+1) = filtYaw;

yawAngle(end+1)    = angle;

counter(end+1)     = count;

packetTime(end+1)  = currentPacketTime;

%% Autosave while running

if mod(length(counter), autosaveEverySamples) == 0

save(autosaveFile, ...

'counter', ...

'yawRate', ...

'filteredYaw', ...

'yawAngle', ...

'packetTime');

disp(['Autosaved to ', autosaveFile]);

end

%% Plot using MATLAB receive timestamp

plotTime = packetTime - packetTime(1);

subplot(4,1,1)

plot(plotTime, yawRate, 'b', 'LineWidth', 1.5)

title('Raw Yaw Rate')

ylabel('deg/s')

grid on

subplot(4,1,2)

plot(plotTime, filteredYaw, 'g', 'LineWidth', 1.5)

title('Filtered Yaw Rate')

ylabel('deg/s')

grid on

subplot(4,1,3)

plot(plotTime, yawAngle, 'r', 'LineWidth', 1.5)

title('Yaw Angle')

ylabel('deg')

grid on

subplot(4,1,4)

plot(plotTime, counter, 'k', 'LineWidth', 1.5)

title('Counter')

ylabel('Count')

xlabel('Time (s)')

grid on

drawnow;

else

pause(0.01);

%% Stop and save only if no data for 20 seconds

if toc(lastDataTime) > 20

disp("Robot disconnected or no data received.");

saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, "final");

break;

end

end

catch ME

disp("Connection error occurred:");

disp(ME.message);

saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, "final");

break;

end

end

%% Cleanup TCP connection

clear t;

disp("Script finished.");

%% Function to save yaw data

function saveYawData(counter, yawRate, filteredYaw, yawAngle, packetTime, mode)

if isempty(counter)

disp("No valid data received. Nothing useful to save.");

return;

end

timestamp = datestr(now, 'yyyymmdd_HHMMSS');

if mode == "final"

filename = sprintf('yaw_data_%s.mat', timestamp);

else

filename = 'yaw_data_autosave.mat';

end

save(filename, ...

'counter', ...

'yawRate', ...

'filteredYaw', ...

'yawAngle', ...

'packetTime');

disp(['Data saved as: ', filename]);

end