Project Summary Document

Pixy2-Based Line Following Robot with Orientation Sensing

1. Introduction

This project involves developing an autonomous line-following robot using the Waveshare AlphaBot platform. The system integrates a camera-based vision sensor for line tracking and an inertial measurement unit (IMU) for orientation sensing. The objective is to enable the robot to follow a path while simultaneously monitoring its heading and rotational movement.

2. Hardware Components

The system is built using the following key components:

- AlphaBot robot chassis with motor driver

- Pixy2 Camera for vision-based line detection

- GY-85 for orientation and motion sensing

- Arduino-compatible microcontroller

- DC motors with PWM control

3. Software Implementation

3.1 Motor Control

- Motor driver pins were configured for both left and right motors.

- Direction logic was implemented to ensure correct forward motion.

- PWM signals are used to control motor speed.

- A dedicated motor control function (setMotor) manages speed and direction.

3.2 Pixy2 Line Tracking

- The Pixy2 camera is initialized in line tracking mode.

- The system retrieves detected line vectors from the camera.

- The center of the detected line is calculated and compared to the camera’s center.

- The difference (error) is used to adjust motor speeds.

- A proportional control approach is used to steer the robot along the line.

3.3 Handling No Line Condition

- A condition is implemented to detect when no line is present.

- When no line is detected:

  - Motor speeds are set to zero

  - The robot stops immediately

- This prevents unintended movement and ensures safe operation.

3.4 GY-85 IMU Integration

- The GY-85 module combines multiple sensors, including:

  - Accelerometer

  - Gyroscope

  - Magnetometer

- It communicates via I2C (depending on the specific chip used inside the module).

- In this project, orientation data is derived primarily from the magnetometer and/or gyroscope components.

- The system computes:

  - Heading (orientation in degrees)

  - Yaw rate (rate of rotation over time)

- Signal processing techniques such as:

  - Deadband filtering (to ignore small fluctuations)

  - Low-pass filtering (to smooth noisy readings)
are applied to improve stability.

3.5 Combined System Behavior

- The Pixy2 camera handles navigation by detecting and following the line.

- The GY-85 IMU provides continuous orientation and motion data.

- Both systems operate simultaneously in the main loop.

- Serial Monitor outputs include:

  - Line detection status

  - Heading / orientation

  - Yaw rate

  - Error values used for steering

4. Issues Encountered and Solutions

4.1 Robot Moving in Reverse

- Caused by incorrect motor direction logic.

- Resolved by reversing motor control signals.

4.2 Motors Running When No Line Detected

- Caused by missing stop condition.

- Fixed by explicitly stopping motors when no line is detected.

4.3 Missing IMU Output

- Caused by early loop exits preventing IMU code execution.

- Fixed by ensuring IMU processing runs every loop iteration.

5. Final System Capabilities

- Autonomous line following using camera vision

- Immediate stopping when the line is lost

- Real-time orientation tracking using GY-85

- Yaw rate calculation and smoothing

- Continuous Serial Monitor feedback

- Stable and corrected motor control

6. Conclusion

The project successfully integrates vision-based navigation with inertial sensing to create a more advanced robotic system. The robot can follow a path autonomously while simultaneously measuring its orientation and rotational movement using the GY-85 IMU. This forms a strong foundation for future improvements such as sensor fusion, PID-based control, and advanced autonomous navigation.